High-performance door Albany RR600

Description:

High-performance door, opening in vertical direction, based on a modular concept for all kind of industrial application from interior use up to rugged, demanding exterior positions.

Self-supporting side frames made of anodised aluminium with integrated cable channel on the back and hinged front covers offering easy access for service and maintenance. The door is equipped with a combined balancing and tensioning system offering smooth operation of the door in demanding environments.

Door curtain made of extremely stiff and durable RollTex® Original – colour

blue (similar to RAL 5002),

orange (similar to RAL 2009) or

grey (similar to RAL 7001).

Wind resistance: up to class 3 (EN 12424), depending on application and configuration.

An innovative bottom beam stopper system (BBS) secures the door when closed.

Bottom profile: anodized aluminium.

Drive unit: 3 phase motor with hollow shaft gear box.

Protection: IP 55

Control system:

Albany MCC, with vector control technology and boost function at start-up. Graphic display and foil keypad for easy, guided operation. Easy access to all important information for operation and service.

control voltage 24 V DC

Supply voltage:

3L/(N)PE 380/400/415/440/480V; 50/60 Hz

Transformator necessary for 220/230/500V; 50/60 Hz.

Safety devices:

electrical safety contact edge with cable chain integrated in side columns,

door line photocell (transmitter/receiver),

drop down protection by integrated balancing system.

Manual activation: by releasing the brake at ground level by means of a bowden lever.

Opening / closing speed: up to max. 3.2 / 0.8 m/s

Size max.: (DWxDH) 6000 x 6000 mm

Make: Albany RR600

Options

Cover:

Top roll cover

Motor cover

Rope pulley cover

Door curtain (alternatively):

- Curtain made of transparent PVC with coloured reinforcement stripes – colour

Beige (similar RAL 1014)

Yellow (similar RAL 1021)

Orange (similar RAL 2011)

green (similar RAL 6018)

white (similar RAL 9016) or

grey(similar RAL 7035)

- PVC with silicone-free reinforcement stripes

- Curtain made of RollTex® Plus – colour

Blue (similar RAL 5002)

orange (similar RAL 2009) or

grey (similar RAL 7001)

- curtain made of RollTex® original – colour

blue (similar RAL 5002)

orange (similar RAL 2009) or

grey (similar RAL 7001)

- Curtain made of NomaTex® (max. door width 5000 mm, max. surface 20m2) – colour

Grey (similar RAL 7038)

- Window DWxDH 500x700mm (only for RollTex® or NomaTex®)

- Window DWxDH 700x500mm (only for RollTex® or NomaTex®)

- Curtain made of NomaTex® flame retardant without vision panels or window – colour grey (similar RAL 7045)

Curtain without wind break taps (only wind class 1).

Miscellaneous

- SRD self repairing knock out capability

- Knock out capability with automatic reset by activating the “close” button

- Patented contact-free, pre-running safety photocell

- Top roll felt sealing to reduce drafts

- Brush sealing in the sideframe

- Yellow sealing bag on the bottom profile to improve sealing on unlevelled floors:

- Light curtain integrated in side frame

- Active infrared sensor integrated in top roll cover

- Locking mechanism for side frame covers offering easy service access

- Actuators

- Additional safety devices

Delivery:

Installation incl. wiring