High Performance Door Albany RR300 Plus

Description:

Flexible High Performance Door, opening in vertical direction for interior use in wind-protected areas.

Self-supporting side frames made of stainless steel

Door curtain made of transparent PVC with fabric reinforcement stripes, ultramarine (similar to RAL 5002) or red (similar to RAL 3002).

Wind protection: through wind tabs on the door curtain.

Wind resistance: class 1 (EN 12424).

Bottom profile: stainless steel V2A.

Drive unit: Hollow shaft gear motor.

Protection: IP 55

Control system: ACS 50, electric control system with mini contactors including main switch and open-stop-close switch, control voltage 24 V DC.

Supply voltage:

ACS 50: 3L(N)PE/230/380/400/415V, 50Hz

Transformator necessary for 440/480/500V 50 Hz.

Safety devices:

electrical safety contact edge,

door line photocell,

drop down protection by integrated balancing system.

Manual activation: by releasing the brake at ground level. Depending on width/height ratio the door opens up to max. 2.0 m.

Opening / closing speed: up to max. 1.2/1.2 m/s.

Size max.: (DWxDH) 4000 x 4200 mm.

Make: Albany RR300 Plus

Options

Cover

Top roll cover required for doors DH < 2500 mm

Motor cover required for doors DH < 2500 mm

Door curtain (alternatively):

- PVC with coloured reinforcement stripes

Beige (similar RAL 1014),

Yellow (similar RAL 1021),

Orange (similar RAL 2011),

Green (similar RAL 6018),

White (similar RAL 9016) or

Grey (similar RAL 7035).

- PVC with silicone-free reinforcement stripes – colour

Blue (similar RAL 5010),

Alu-grey (similar RAL 7037),

Red (similar RAL 3002),

Yellow (similar RAL 1021),

Orange (similar RAL 2011),

White (similar RAL 9010),

Black (similar RAL 9005).

- Curtain made of RollTex® Plus – colour

blue (similar RAL 5002),

orange (similar RAL 2009)or

grey (similar RAL 7001)

- Curtain made of RollTex® Original – colour

blue (similar RAL 5002),

orange (similar RAL 2009)or

grey (similar RAL 7001 varnishable on request)

- Curtain made of NomaTex® – colour grey (similar RAL 7038)

- Curtain made of fabric type travitop colour blue, red or grey

- Window (DWxDH) 500x700mm (only for RollTex® or NomaTex®)

- Window (DWxDH) 700x500mm (only for RollTex® or NomaTex®)

- Vision panels for curtain NomaTex®

- Curtain made of NomaTex® flame retardant without vision panels or window – colour grey (similar RAL 7045)

- Curtain made of non-transparent fabric, type travitop – colour

Blue (similar RAL 5010),

Red (similar RAL 3002) or

Grey (similar RAL 7035)

- Vision panels for curtain type travitop

Control sysem (alternatively):

Frequency converter control system Albany MCC, based on vector control including boost function at start-up. Graphic display and foil keypad for easy guided operation. Easy access to all important information for operation and service,

(open/close) up to max. 2,2/0,9 m/s,

Supply voltage 3L(N)PE/380/400/415/440/480V;50/60Hz,

Transformer necessary for 220/230/500V:

Miscellaneous:

- Knock-out capability with sensor, only in combination with coil cable (without interior cable chain)

- Integrated light curtain, not possible in combination with knock-out function

- Automatic egress on power failure

- Extended flexible seal

- Actuators

Option: Escape and rescue routes:

Size max.: (DWxDH) 4000 mm x 4000 mm

Wind resistance: class 1 (EN 12424) up to DH = 3500 mm, class 0 (EN 12424 at DH > 3500 mm

Control unit:

Solely frequency converter control Albany MCC with extension and special software (configuration).

Drive unit: hollow shaft gear motor with operating current brake

Emergency opening: In case of power failure the door opens automatically, the door can be opened manually via the main switch/CEE plug.

A radar motion detector for the detection of people is included in delivery.

Key switches for the deactivation of the escape and rescue function is optionally available.

Delivery:

Installation incl. wiring