Crawford Frame Sectional door

Product description:

- Overhead sectional door/sectional door with panels of extruded, E6-EV1, anodized, high- strength aluminium profiles as supporting frame (Height 450 - 700 mm).

- Infills optionally single- or insulating glazing and/or sandwich panels.

- Integrated finger pinch protection

- Top, bottom and side seals and seals between door sections

- Standard track system in galvanized steel

- Spring balancing, 20.000 cycles

- Spring break device

- Lock bolt, step-handle,

- Compliance with all operational and safety instructions in EU Directives and the standards of the European Standardization Committee (CEN)

- Standard hardware (head room 485/510 mm to 800 mm)

- Operating system: manual

- Standard door sizes (DLW x DLH) MIN: 2.000 x 2.000 mm

MAX: 7.250 x 6.000 mm

- Wind load (EN 12424)	Class 3 (700 N/m²) up to door width 4.250 mm,

Class 2 (450 N/m²) up to door width 7.250 mm.

- Thermal transmittance (EN 12428)	3.3 W / (m² K) insulating glazing

4.9 W / (m² K) single glazing

- Water penetration (EN 12425)	Class 3, 70 PA (Door 4.000 x 4.000 mm)

- Air permeability (EN 12426)	Class 2 (Door 4.000 x 4.000 mm + passdoor)

Dimensions:

x	mm (WxH)

Bottom Sections:

- Standard: Aluminium frame construction

- Option: Sandwich panel across entire width, surface waffled, Section height 545 mm plus bottom seal

Infills Glazing:

- Standard: scratch-resistant acrylic sheets, 2 x 3 mm, total thickness 27 mm, single sealed

- Option: acrylic sheets, 2 x 3 mm, total thickness 27 mm, single sealed

- Option: acrylic glass 2 x 3 mm, total thickness 27 mm, double sealed

- Option: scratch-resistant acrylic glass 2 x 3 mm, total thickness 27 mm, double sealed

- Option: single glazing 3 mm acrylic

- Option: scratch resistant single glazing 3 mm acrylic

- Option: single glazing 4 mm hardened glass

- Triple glazing – scratch resistant

- Triple glazing – not scratch resistant

Sandwich units:

- Standard: infill, aluminium units, surface stucco interior / exterior

- Option: infill, steel units, interior / exterior RAL 9006 surface smooth

- Option: infill, aluminium units, surface aluminium stucco interior / exterior smooth

- Option: infill, aluminium units interior / exterior anodized (Other glazing / sandwich unit designs available on request)

Options Surface colour:

- Aluminium frame

  - Standard: E6/EV1 anodized

  - Option: interior and / or exterior painted in standard colours

  - Option: interior and / or exterior painted in RAL-standard colour buyer's choice

  - Option: interior and / or exterior painted according to colour sample

  - Option: interior and exterior powder coated

- Sandwich panel

  - Standard: Aluminium panel aluminium interior and exterior or

  - Standard: Steel panel interior RAL 9002, exterior standard colours

  - Option: Aluminium and steel panel interior and/or exterior painted according to colour sample

  - Option: Aluminium and steel panel interior and / or exterior painted in RAL-standard colours buyer's choice

- Infills

  - Standard: Design see above

  - Option: interior and / or exterior painted in standard colours

  - Option: interior and / or exterior painted in RAL-standard colour buyer's choice

  - Option: interior and / or exterior painted according to colour sample

  - Option: interior and exterior powder coated

- Passdoor

  - Standard:

Opening outwards, door seals, Left or Right hinged, PC lock, doorway clearance

depending on height of panels, doorway clearance width 900 mm, overhead door closer, anodized profile of passdoor, glazing and infills like rest of door, passdoor safety switch

  - Option: clearance width passdoor between 750 - 1.000 mm

  - Option: PC lock with knob cylinder

  - Option: Nemef panic lock

  - Option: Nemef panic lock with panic bolt

Door locking device:

- Option: Second lock bolt

- Option: Lock with profile cylinder lockable on one or both sides

- Option: Door locking switch (essential with electric operation)

Hardware:

- Standard Low Lift (drop clearance 400 mm to 485/510 mm)

- Low Lift (drop clearance 265/310 mm to 485/510 mm)

- High Lift (drop clearance 800 mm to door height + 390 mm) - maximum lift 3.900 mm

- Vertical Lift (drop clearance min. door height + 400 mm)

Operator:

- Option: Chain hoist "T" (Gear ratio 1:4, Door weight max. 250 kg)

- Option: Chain hoist "U" (Gear ratio 1:3, Door weight over 250 kg)

- Option: Electronic operator:

- Operating voltage: 230 V AC, 50/60 HZ (internal supply 3-phase 230 V AC)

- Operating factor: ED = 30%, S3 10 min, non-continuous operation

- (Engine operation 3 min, pause 7 min, direct-inverting)

- Torque: 15-40 Nm, depending on reel type (value of given door shaft)

- Fusing: 10 A

- Brake: non-reversible worm gear

- Speed monitor: electronic monitoring

- Freewheel / clutch: mechanical engagement/disengagement

- Limit switch: electronic via torque (setting "Door open" during programming) Setting "Door closed" in processor board

- Gears: worm gear 1750 RPM yields approx. 24 rotations door shaft

- Shaft: ø35 mm or hexagonal

- Direct drive system

- No special wall reinforcement necessary

- Temperature range: -20°C to 60°C

- Soft stop, soft start

- Connection using "CEE plug" or lockable mains isolator

- Maximum door weight 400 kg

Options:

  - Chain transmission

  - Chain guard

Other drive variants available on request subject to technical inspection

Control units:

- Control unit

Output 0.37 kW, input 1 x 230 V. Including control unit with standard actuator UP-STOP- DOWN. The door closes using dead-man controls.

Operating speed 0.25 m/s, safety rating IP 55. If there is a power failure, the disengage device enables manual operation at any time

Options:

  - Additional buttons, key switch

  - Door/bridge interlock

- Control unit

Output 0.37 kW, input 1 x 230 V. Including control unit with standard actuator UP-STOP- DOWN. The door closes using pulse control incl. self-monitoring safety edge

Operating speed 0.25 m/s, safety rating IP 55. If there is a power failure, the disengage device enables manual operation at any time

Options:

  - Radio remote control

  - "Open" (if open button is used during closing process - door immediately stops and opens completely)

  - "Automatic close" (safety photo cell compulsory requirement)

  - Door closes automatically after set period (4-60 sec)

  - Pull switch for opening function

  - External push-button, key switch

  - Door/bridge interlock

- Control unit

Output 0.37 kW, input 1 x 230 V. Dimensions: 180 x 380 x 120 mm (wxhxd) including control unit with standard actuator UP-STOP-DOWN and pulse control incl. self- monitoring safety edge. Counter/error code display and servicing info display in top of control cabinet. Operating speed 0.25 m/s, safety rating IP 55. If there is a power failure, the disengage device enables manual operation at any time.

Options:

- Radio remote control

- "Open" (if open button is used during closing process - door immediately stops and opens completely)

- "Automatic close" (safety photo cell compulsory requirement) Door closes automatically after set period (4-60 sec)

- Pull switch for OPEN / CLOSE

- External push-button, key switch

- "One-button function" - opens or closes door, depending on actual status (e.g. radio, pull switch)

- additional safety (e.g. photo cell, radar) – door immediately opens completely

- D-Module (D1-D7)

- Photo cell

- Reduced opening height

- Radar

- Magnetic loop

- Warning lights

- Traffic lights

- Interlocking door function

- Door/bridge interlock

D-Module (automation module)

- D1 functions

- red lamps, inclusive, blinking continuously when door is moving

- red blinking light before "Automatic close"

- constant red light with door closed available

- Start time for blinking lights before closure can be set

- D7 - function incl.

- D2 functions

- Magnetic loop, inclusive, with various functions, e.g.

  - "Safety and checkback signal"

  - "Open and checkback signal"

  - "Safety and open"

  - "Open"

- A DIP switch can also be chosen to provide confirmation as additional safety

- D7 - function incl.

- D3 - functions

- Interlocking door function or mutual interlocking between two doors

- Door/bridge interlocking

- Incl. 24 V AC max. 400 mA for additional units

- Relay for "Door open" or "Door closed"

- D4 functions

- Combination of D1 and D2

- red lamps (inclusive) blinking continuously when door is moving

- red blinking light (inclusive) before "Automatic close"

- constant red light with door closed available

- Start time for blinking lights before closure can be set

- A DIP switch can also be chosen to provide confirmation as additional safety

- D7 - function incl.

- magnetic loop (inclusive) with various functions, e.g.

  - "Safety and checkback signal"

  - "Open and checkback signal"

  - "Safety and open"

  - "Open"

- D5 functions

- Directional sensor traffic light (including red/green)

- Control of oncoming traffic in the doorway

- using "Open interior" or "Open exterior" to control oncoming traffic

- Oncoming traffic sees alternately red or green light, depending on which "Open command" is given first

- D3 - functions

- D6 functions

- Directional sensor traffic light with magnetic loop activation (inclusive), of "Open exterior"

- Control of oncoming traffic in the doorway

- using "Open interior" or "Open exterior" to control oncoming traffic

- Oncoming traffic sees alternately red or green light, depending on which "Open command" is given first

- D3 - functions

- D7 functions

- Interlocking door function or mutual interlocking between two doors

- Door/bridge interlocking

- Incl. 24 V AC max. 400 mA for additional units

- Relay for "Door open" or "Door closed"

Other supplementary equipment

- Anti-corrosive hardware

  - Anti-corrosive package Type "C"

    - Stainless steel hinges and roller brackets

    - Stainless steel screws

    - Stainless steel roller shafts and door cables

    - Aluminium top and bottom profile

    - Powder coated corner bracket (RAL 9002)

  - Anti- corrosive package Type "A"

    - Stainless steel hinges and roller brackets

    - Stainless steel screws

    - Stainless steel roller shaft and door cables

    - Aluminium top and bottom profile

    - Powder coated corner bracket (RAL 9002)

    - Powder coated end caps, trusses, tracks, hangers, connection beams, safety pull bracket (RAL 9002)

  - Anti- corrosive package Type "S"

    - Safety pull cables, stainless steel

  - Anti- corrosive package Type "Z"

    - Electro-galvanized springs (max. 70.000 cycles)

- Cable break device

- Increased cycle spring (20.000 – 100.000)

- Ventilator "P" (free area approx. 170 cm²)

Special models

- Fixed top and side sections, with and without passdoor, in the same colour and construction of the door leaf are available on request !! (subject to technical inspection)