Aluminium High-performance door Albany RR3000 R

Description:

Aluminum High-performance door, opening in vertical direction, for interior and exterior use in locations with reduced space required over the lintel, suitable as laser and machine protection door

Self-supporting edged side frames of galvanized steel profiles, all steel components galvanized.

Door curtain of ecru anodized aluminum lamellas, hingeless connected by means of flexible heavy belts, wear-less to wind up without contacting each other, each single lamella can be replaced easily, pre-fitted on the top roll.

Wind resistance: class 3 - 4 (EN 12424).

Drive unit: gear motor, pluggable electrical connections.

Protection: IP 55

Control system:

Albany MCC, with vector control technology and boost function at start-up, graphic display and foil keypad for easy guided operation. Easy access to all important information for operation and service

Control voltage 24 V DC.

Supply voltage:

3L/(N)PE 380/400/415/440V; 50/60 Hz

Transformator necessary for 230/480/500V 50/60 Hz.

Safety devices:

contact-free pre-running safety photocell (transmitter/receiver)

door line photocell (transmitter/receiver)

drop down protection by integrated balancing system

Manual activation: by releasing the brake at ground level. Partly opening by means of the pre-stressed tension springs.

Space required over the lintel: max. 500 mm including fixing space.

Opening / closing speed: up to max. 2.0 / 0.6 m/s

Size max.: (DW x DH): 6000 x 3000 mm

Make: Albany RR3000 R

Options

Covers:

- Top roll cover made of anodized aluminum

- Motor cover made of anodized aluminum

Surface/material:

- Side frames with covers made of anodized aluminum

- Side frames coloured

- Aluminum lamellas coloured

Miscellaneous

- Aluminum lamellas with window

- insulation

- extended flexible seal

Actuators

Additional safety devices

Delivery

Installation incl. wiring