Crawford VL3116 vertical lifting fabric door

Product description:

- Vertical lifting fabric door designed for industrial environments where doors are exposed to moisture, dust, very high or low temperatures or where the door opening is large.

- The unique design and structure offers durability, tightness, energy efficiency, operational reliability and minimum maintenance.

Door leaf

- Door leaf made out of two layers of very strong vinyl-coated polyester fabric, separated by aluminium intermediate sections.

- Intermediate section which gives strength to the door leaf and which creates a buffer between the inside and outside fabric walls.

- Wind load is transferred to the vertical guide rails by the horizontal aluminium sections of the door leaf.

- The door leaf is available in 9 standard colours.

- Door leaf thickness: 160 mm.

- Aluminium guide rails.

Bottom section

- The bottom section, made of steel and aluminium, is connected to the lifting belt via the safety arresters.

- The bottom section contains an optical safety edge and a rubber seal on the bottom edge that creates a seal between the door and the floor.

- Optical safety edge.

Safety arresters and wind lock in stainless steel

- Safety arresters stop the door if the lifting belt becomes slack or, in an unlikely event, would break.

- Wind lock system locks the door when the door is closed.

Surface treatment

- Steel components corrosion treatment: category 3 according to ISO 12944.2. Higher class on request. Aluminium, plastic, stainless steel, zinc electroplated steel (~10u). Fixing elements are mainly hot dip galvanized (FZV). Door leaf screws protected with Geomet.

Standard door features

- Door range:

MIN: 6000mm x (no MIN. height) MAX: 14000 x 16000 mm

- Normal operating speed of 0.2 - 0.3 m/sec

- Temperature range: -35 °C to +70 °C

- Compliance with all operational and safety instructions in EU Directives and the standards of the European Standardisation Committee (CEN)

- Wind load (EN 12424)	Class 2-5, 0.45-1.6 kPa, depending on size

- Water resistance (EN 12425)	Class 3, 0.11 kPa (for a closed door)

- Air permeability (EN 12426)	Class 2, 12 m3/(m2h)

- Thermal transmittance (EN

12428)

Depending on door size. Specific data on request.

- Sound reduction (ISO 717)	15 dB Rw

- Operating system: Electrical operator

Dimensions:

x	mm (WxH)

Options:

Door leaf

- Standard: Polyester, 1100 dtex with plasticised PVC coating

- Option: Arctic fabric. For environmental temperatures down to -54°C

- Option: Heat resistant fabric with silicone rubber coating

- Option: Heat resistant fabric with aluminium coating

- Option: Heat resistant fabric with aluminium polyurethane coating

- Option: Sound reduction fabric

- Option: Security fabric

Vision panels

- Width 800 mm, height between the intermediate profiles (depending on door size)

- Width 1300 mm, height between the intermediate profiles (depending on door size)

Fabric color

- Beige (RAL 1001)

- Red (RAL 3001)

- Blue (RAL 5005)

- Green (RAL 6009)

- Grey (RAL 7004)

- Anthracite (RAL 7016)

- White (RAL 9016)

- White aluminium (RAL 9006)

- Translucent white

Header box

- Gear motor on inside of the building

- Gear motor on outside of the building

- Gear motor in the door opening

- Insulated

- Stainless steel cladding and motor casing

- Self-supporting header box

Operator

- Control system PLC-based

- Protection class, control cabinet IP65

- Protection class, limit switches IP67

- Protection class, motor brake IP55

- Protection class, push buttons IP65

- 3/phase 400V 50H power supply

- Control voltage 24DC

- Fusing: 20 A

- Heat and cold resistance: -35 °C to +70 °C

- Motor ratings: 2.8 - 5.0 kW

Options

- Free contacts: 6 for control of user functions

- Radio remote control

- “Open" (during closing process) if open button is used - door immediately stops and opens completely)

- Pull switch for OPEN / CLOSE

- External push-button, key switch

- “One-button function” - opens or closes door, depending on actual status (e.g. radio, pull switch)

- Additional safety (e.g. photo cell, radar)

- Photo cell

- Reduced opening height

- Radar

- Magnetic loop

- Warning lights

- Traffic lights

- Interlocking door function

Other supplementary equipment

- Control cabinet in stainless steel

- Heating element in the control cabinet

- Emergency power switch with separate steel casing

- Clamp strip covers in the same colors as the fabric

- Side jambs (if posts for installing the guide rails are not available)

- Side jambs in stainless steel

- Wind deflectors (in combination with jambs)