Crawford DL6020T teledock

Product description:

Stationary dock leveller with telescopic lip with:

- Integrated frame with wall anchors (T-frames)

- Electro-hydraulic operation with two lifting cylinders

- One hydraulic cylinder for infinitely-adjustable extension of steel lip

- Automatic, free floating position

- Three-button switch with standard main switch, incl. contact to lock door / leveller

  - Hold-to-run button to lift platform

  - Hold-to-run button to position the lip on the truck bed

  - Impulse auto button to put leveller back in parking position

- Hydraulic emergency stop in any operating position

- Automatic mechanical locking in neutral position

- Lateral torsion flexibility

- Side plates (toe guards) with warning signs both above and below dock level

- Internal wiring, plug connection

- Power supply by others

- Control Unit Protection Class IP54

- The dock leveller meets standard demands of most loading operations and fully complies with rules and regulations of the European Standard EN 1398

Dimensions:

Dimensions	Vertical working range 60 kN

LE 500

LE 1000	LE 345

| NL | LH | A | B | A | B | A | B |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2000 | 600 | 310 | 310 | 380 | 370 | 270 | 280 |
|  | 700 | 340 | 400 | 420 | 470 | 370 | 360 |
| 2500 | 600 | 420 | 260 | 500 | 290 | 380 | 225 |
|  | 700 | 430 | 370 | 510 | 420 | 400 | 340 |
| 3000 | 600 | 370 | 230 | 430 | 250 | 350 | 210 |
|  | 800 | 450 | 400 | 520 | 440 | 420 | 360 |
| 3500 | 800 | 520 | 400 | 600 | 440 | 490 | 360 |
| 4000 | 900 | 590 | 400 | 660 | 440 | 560 | 380 |
| 4500 | 900 | 570 | 410 | 640 | 440 | 550 | 380 |

Nominal width (NW) 1750, 2000, 2200, 2250mm for all sizes.

Surface:

- Surface painted in standard colour RAL 5010, thickness 80 µm

Lip bearing:

- Reinforced steel tear plate swing lip

- 500 mm telescopic lip

- 40 mm bevelled lip

Hydraulics:

- Standard oil for temperature range -20°C - +60°C

Control unit

- Voltage 400V 3-phase, 230V 3-phase, cable harness length 7m

- Display of error messages

- Service interval display

- About function / Memory function

- Impulse auto button to put the leveller back in parking position

Options:

Surface

- Surface painted in RAL 9005, RAL 6005, RAL 3002, thickness 80 µm

- Surface painted with coat thickness 160 µm

- Surface hot-dipped galvanised

Frame

- W-Frame (welded to pre-cast frame)

- F-Frame (flat bar frame welded to existing frame or for replacement)

- B-Frame (box frame, for positioning dock leveller and subsequent setting in concrete)

- P-Frame (pit frame with support from under (made by others), up to NL=3000mm)

Hydraulics

- Low temperature oil for temperature range -30°C - +60°C

- Bio-oil (environmental protection)

Bridge bearing

- Steel lip with front sill of 100mm

- Lip in aluminium

- Lip with oblique section 100mm

- Lip with retracting tongues in steel, 1 per side (140mm)

- Lip 1000mm

Energy & Ergonomics

- EPDM seal

- Slip protection and noise reduction coating

- Front cover curtain PVC (up to nominal length 3000mm)

Multifunctional docking control for direct control of dock leveller, dock shelter and door with just one control unit

- Operating dock leveller and door

- Operating dock leveller dock shelter and door

- Operating dock leveller and dock shelter

Control unit

- Request parking position

- Voltage 400V 3-phase, 230V 3-phase

- Cable harness extension

- External EMERGENCY STOP button