Crawford DL6020TI teledock isodock

Product description:

The DL6020TI teledock isodock is the optimal energy saving solution for loading and unloading in temperature controlled supply chains. The telescopic lip precisely bridges the gap between the ramp and the lorry bed. The leveller is provided with a polyurethane filled insulation construction that achieves an energy cost reduction of up to 75%.

- Integrated frame with wall anchors (T-frames)

- Electro-hydraulic operation with two lifting cylinders

- One hydraulic cylinder for infinitely-adjustable extension of lip

- Automatic, free floating position

- Three-button switch with standard main switch, incl. contact to lock door / leveller

  - Hold-to-run button to lift platform

  - Hold-to-run button to position the lip on the truck bed

  - Impulse auto button to put leveller back in parking position

- Hydraulic emergency stop in any operating position

- Automatic mechanical locking in neutral position

- Lateral torsion flexibility

- Side plates (toe guards) with warning signs both above and below dock level

- Internal wiring, plug connection

- Power supply by others

- Control Unit Protection Class IP54

- The dock leveller meets standard demands of most loading operations and fully complies with rules and regulations of the European Standard EN 1398

- EPDM seal all-round on three sides

Dimensions:

Dimensions	Vertical working range

| NL | LH | A | B | PD |
| --- | --- | --- | --- | --- |
| 2000 | 830 | 380 | 460 | 850 |
| 2500 | 830 | 450 | 450 | 850 |
| 3000 | 900 | 520 | 450 | 920 |

Nominal width (NW) 2000, 2200 for all sizes.

Load bearing capacity : 6 tonnes (60kN)

Surface:

- Surface painted in standard colour RAL 5010, thickness 80 µm

Lip bearing:

- Reinforced steel tear plate telescopic lip

- Steel lip length 1000mm

- Tapered lip 40mm

Hydraulics:

- Standard hydraulic oil for temperature range -20°C - +60°C

Control unit

- Voltage 400V 3-phase, 230V 3-phase, cable harness length 7m

- Display of error messages

- Service interval display

- Readout function / Memory function

- Impulse auto button to put the leveller back in parking position

Options:

Surface

- Surface painted in RAL 9005, RAL 6005, RAL 3002 thickness 80 µm

- Surface painted with coat thickness 160 µm

- Surface hot dip galvanized

Hydraulics

- Low temperature oil for temperature range -30°C - +60°C

- Bio-oil (environmental protection)

Lip bearing

- Steel lip with front sill of 100 mm

- Lip extension in aluminium

- Lip extension with oblique section 100mm

- Lip extension with retracting tongues in steel, 1 per side (140mm)

Energy & Ergonomics

- Slip protection and noise reduction coating

Multifunctional docking control for direct control of dock leveller, dock shelter and door with just one control unit

- Operating dock leveller and door

- Operating dock leveller dock shelter and door

- Operating dock leveller and dock shelter

Control unit

- Request parking position

- Voltage 400V 3-phase, 230V 3-phase

- Cable harness extension

- External EMERGENCY STOP button